- How to Upgrade CS100/CS101 Two Way Radio

- Run CFU_V1.01a.exe to install the upgrading tools.

- Then run the upgrading software, such as duouble clicking the shortcut icon.

- Choose Radio Type as CU600, set the right serial port and baud rate

- Open the *.bin file, and connect the programming cable to the wanted radio

- Power on the radio, and then click ”E.P” (Erase, Program).

! Pay attention to keeping the cable connected stably and the radio is power on all the upgrading period.

NOTE： CU500 firmware version

- After reading the radio,If the firmware version starts with ‘E’, you should choose CU500_Exxx.bin; if starts with ‘S’,you should choose CU500_Sxxx.bin

- After reading the radio, If the firmware is  (E/S)1.1.xx or less, you should choose the bin file with xx, or none.